UNIVERSITY OF GHANA

School of Engineering Sciences

Department of Computer Engineering

FINAL PROJECT REPORT

Part B: 32-bit Single-Cycle MIPS Processor Components in VHDL

First Semester 2025/2026 Academic Year

Submission Date: 3rd April 2026

Group 13

- Jefferson Adda

- Herbert Damoah

1. Abstract

This report documents the design and implementation of the key components of a 32-bit single-cycle MIPS processor using VHDL (VHSIC Hardware Description Language). The project is Part B of the CPEN 315 Final Project and extends the conceptual 16-bit CPU studied in class to a full 32-bit architecture. Five core components were implemented: the Data Memory, the Arithmetic Logic Unit (ALU), the Register File, the Instruction Memory, and the Main Control Unit. Each component was verified through dedicated VHDL testbenches using the GHDL simulator, with waveform outputs captured in Value Change Dump (VCD) format. All five task groups specified in the project brief were completed successfully, including memory read/write operations, arithmetic and logical computation, register file access, instruction fetch, and control signal generation for all eight supported opcodes.

2. Introduction

The MIPS (Microprocessor without Interlocked Pipeline Stages) architecture is one of the foundational RISC (Reduced Instruction Set Computing) designs in computer architecture education. Understanding its internal structure provides insight into how modern processors execute programs at the hardware level.

This project targets a 32-bit single-cycle MIPS processor, where every instruction completes execution within a single clock cycle. The design is structured around a datapath comprising five major subsystems: instruction memory for fetching instructions, a register file for temporary data storage, an ALU for arithmetic and logical computation, data memory for load/store operations, and a control unit for decoding opcodes and asserting the correct datapath signals.

All components were described in VHDL and simulated using GHDL, an open-source VHDL simulator. The VS Code IDE was used as the development environment, with a tasks.json configuration enabling one-click compilation and simulation. The objectives of this project were to:

- Gain proficiency in VHDL hardware description for processor components.

- Implement all five 32-bit processor components to specification.

- Validate each component through comprehensive testbenches.

- Analyse and discuss simulation waveforms for correctness.

3. Architecture Design

## 3.1 Instruction Set Architecture (ISA)

The processor supports a subset of the MIPS ISA with three instruction formats: R-format (register-to-register operations), I-format (immediate and memory operations), and J-format (unconditional jumps). Given that our register file contains 8 registers, register addresses are encoded in 3 bits instead of the standard 5 bits. The instruction word is 32 bits wide.

| Format | Fields | Instructions Supported |
| --- | --- | --- |
| R-format | op(3) ¦ rs(3) ¦ rt(3) ¦ rd(3) ¦ shamt(3) ¦ funct(3) ¦ pad(14) | add, sub, and, or, slt, jr |
| I-format | op(3) ¦ rs(3) ¦ rt(3) ¦ immediate(23) | lw, sw, beq, addi, slti |
| J-format | op(3) ¦ target address(29) | j, jal |

## 3.2 Opcode Table

| Opcode (3-bit) | Instruction | Type | Key Action |
| --- | --- | --- | --- |
| 000 | R-type (add, sub, and, or, slt) | R | ALU op from funct field |
| 001 | lw | I | Load word from memory |
| 010 | sw | I | Store word to memory |
| 011 | beq | I | Branch if equal (SUB + zero flag) |
| 100 | addi | I | Add immediate to register |
| 101 | j | J | Unconditional jump |
| 110 | jal | J | Jump and link (save PC+4 to R7) |
| 111 | slti | I | Set less than immediate |

## 3.3 ALU Operations

The ALU is driven by a 3-bit ALU control signal generated by the ALU Control Unit, which combines the 2-bit ALUOp from the main Control Unit and the 3-bit funct field from R-type instructions.

| ALU Control | Operation | Description |
| --- | --- | --- |
| 000 | ADD | Signed 32-bit addition |
| 001 | SUB | Signed 32-bit subtraction |
| 010 | AND | Bitwise logical AND |
| 011 | OR | Bitwise logical OR |
| 100 | SLT | Set result=1 if A < B, else 0 |

## 3.4 Control Unit Signal Table

The main Control Unit decodes the 3-bit opcode and generates the following 10 control signals for the datapath:

| Instruction | reg_dst | mem_to_reg | alu_op | jump | branch | mem_rd | mem_wr | alu_src | reg_wr | sign_ext |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-type | 01 | 00 | 10 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| lw | 00 | 01 | 00 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| sw | 00 | 00 | 00 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| beq | 00 | 00 | 01 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| addi | 00 | 00 | 11 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| j | 00 | 00 | 00 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| jal | 10 | 10 | 00 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| slti | 00 | 00 | 10 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |

4. Implementation Details

## 4.1 Development Environment

- IDE: Visual Studio Code with the VHDL extension (puorc)

- Simulator: GHDL v4.0.0 (open-source VHDL simulator)

- Waveform Viewer: GTKWave

- Hardware: Apple MacBook Air (M3), macOS

- VHDL Standard: IEEE VHDL-2008 (--std=08)

A .vscode/tasks.json file was configured with tasks to compile all source files, compile all testbenches, and run each testbench individually or all at once, producing VCD waveform files in a dedicated waveforms/ directory.

## 4.2 Data Memory (data_memory.vhd)

The Data Memory implements a 256-entry synchronous RAM, where each entry is 32 bits wide. The memory address is derived from the lower 8 bits of the 32-bit address input, giving a byte-addressable range of 256 locations. Both read and write operations are synchronous, triggered on the rising edge of the clock.

Write behaviour: when mem_write_en is asserted, the value on mem_write_data is stored at the address on the rising clock edge. Read behaviour: when mem_read is asserted, the stored value at the given address is driven onto mem_read_data on the rising clock edge; otherwise the output is forced to 0x00000000.

Key port declarations:

clk             : in  STD_LOGIC;

mem_access_addr : in  STD_LOGIC_VECTOR(31 downto 0);

mem_write_data  : in  STD_LOGIC_VECTOR(31 downto 0);

mem_write_en    : in  STD_LOGIC;

mem_read        : in  STD_LOGIC;

mem_read_data   : out STD_LOGIC_VECTOR(31 downto 0);

## 4.3 Arithmetic Logic Unit (alu.vhd)

The 32-bit ALU is a purely combinational component that operates on two 32-bit signed operands A and B. A 3-bit alu_control signal selects one of five operations. A zero flag output is driven high whenever the result equals 0x00000000, which is used by the BEQ instruction to determine whether a branch should be taken.

The ALU is implemented as a process sensitive to alu_control, a, and b. A CASE statement dispatches to the appropriate IEEE.NUMERIC_STD signed arithmetic or bitwise operation.

## 4.4 ALU Control Unit (alu_control.vhd)

The ALU Control Unit is a two-level decoder. It takes ALUOp (2 bits from the main Control Unit) and ALU_Funct (3 bits from the instruction funct field) and produces the 3-bit ALU_Control signal. When ALUOp is 00 (lw/sw), it forces ADD. When ALUOp is 01 (beq), it forces SUB. When ALUOp is 10 (R-type), it passes the funct field directly as the ALU control. When ALUOp is 11 (addi), it forces ADD.

## 4.5 Register File (register_file.vhd)

The Register File contains 8 general-purpose 32-bit registers (R0 through R7). R0 is hardwired to 0x00000000: writes to R0 are blocked, and reads from R0 always return zero regardless of stored content.

The register file provides two asynchronous read ports and one synchronous write port. Reads are combinational and respond immediately to changes in the read address inputs. Writes are registered on the rising clock edge when reg_write_en is asserted. A synchronous reset initialises all registers to non-zero default values (R1=1, R2=2, ... R7=7) to facilitate testbench verification.

## 4.6 Instruction Memory (instruction_memory.vhd)

The Instruction Memory is a read-only ROM with 16 32-bit entries, addressed by the lower 4 bits of the 32-bit program counter. It is a purely combinational component. When the PC value is greater than or equal to 0x00000020 (decimal 32), the output is forced to 0x00000000 (NOP), representing the boundary of the instruction memory.

Four instructions were preloaded for Task 4, using the adapted 3-bit register-field encoding:

- Address 0: add $t0, $t1, $t2

- Address 1: sub $t2, $t2, $t3

- Address 2: and $t1, $t2, $t0

- Address 3: or $t2, $t3, $t1

Register mapping: $t0=R1, $t1=R3, $t2=R5, $t3=R7, consistent with the odd-register convention used in Task 3.

## 4.7 Main Control Unit (control_unit.vhd)

The Control Unit is a combinational decoder that reads the 3-bit opcode and a reset signal and asserts 10 datapath control signals. It is implemented as a CASE statement on the opcode, with a reset condition that deasserts all signals. Each opcode case sets all ten signals simultaneously, ensuring no undriven outputs at any given time.

The ten output signals and their roles are: reg_dst (selects the destination register field), mem_to_reg (routes memory or ALU result or PC+4 to the register write port), alu_op (drives the ALU Control Unit), jump (asserts the PC jump mux), branch (gates the branch adder output), mem_read and mem_write (data memory enables), alu_src (selects the second ALU operand between a register and a sign-extended immediate), reg_write (enables a write to the register file), and sign_or_zero (selects sign extension versus zero extension of the immediate field).

5. Simulation Results and Discussion

All five testbenches were compiled and run using GHDL v4.0.0 with the VHDL-2008 standard. The results below are derived from the VCD waveform data captured during simulation.

## 5.1 Task 1 — Data Memory

The data memory testbench verified synchronous write and read operations at two addresses, followed by a retention check to confirm no memory corruption between operations.

| Operation | Address | Data Written | Data Read Back | Result |
| --- | --- | --- | --- | --- |
| Write then Read | 2 | 1024 | 1024 | PASS |
| Write then Read | 4 | 429496 | 429496 | PASS |
| Retention Check | 2 | — | 1024 | PASS |

The VCD waveform confirms that write data (0x00000400 for 1024 and 0x00068FF8 for 429496) was correctly captured on the rising clock edge and subsequently read back without corruption. The mem_read_data output returned to 0x00000000 between reads when mem_read was deasserted, as expected.

## 5.2 Task 2 — ALU

The ALU testbench applied five test cases covering ADD, SUB, AND, OR, and SLT operations, plus a zero-flag verification using equal operands under subtraction.

| Operation | Operand A | Operand B | Expected Result | Simulated Result | Pass? |
| --- | --- | --- | --- | --- | --- |
| ADD | 2,500 | 25,000 | 27,500 | 27,500 | PASS |
| SUB | 540,250 | 37,800 | 502,450 | 502,450 | PASS |
| AND | 53,957 | 30,000 | 20,512 | 20,512 | PASS |
| OR | 746,353 | 846,465 | 1,046,897 | 1,046,897 | PASS |
| SLT | 58,847,537 | 72,464,383 | 1 | 1 | PASS |
| Zero flag (SUB A=B) | 0xABCD | 0xABCD | zero=1 | zero=1 | PASS |

The VCD data shows alu_result transitions at timestamps 1 ns, 21 ns, 41 ns, 61 ns, and 81 ns, each corresponding to the settling of a new operand pair. The SLT result of 0x00000001 at t=81 ns confirms correct signed comparison. The zero flag transitions to logic-1 at t=101 ns when both operands equal 0xABCD under subtraction.

The AND result deserves brief explanation: 53,957 in binary is 0xD2A5 and 30,000 is 0x7530. Their bitwise AND is 0x5020, which equals 20,512 in decimal, consistent with the simulation output.

## 5.3 Task 3 — Register File

The register file testbench reset the component, wrote four large values to registers R1, R3, R5, and R7, then performed simultaneous dual-port reads to verify stored data. A final read of R0 confirmed the hardwired-zero behaviour.

| Register | Value Written | Value Read Back | Pass? |
| --- | --- | --- | --- |
| R1 | 1,934,858 | 1,934,858 | PASS |
| R3 | 8,558,447 | 8,558,447 | PASS |
| R5 | 203,848,544 | 203,848,544 | PASS |
| R7 | 20,670,420 | 20,670,420 | PASS |
| R0 | N/A (hardwired) | 0 | PASS |

The VCD waveform shows both read ports (reg_read_data_1 and reg_read_data_2) asserting the correct values combinatorially after the read addresses are driven, confirming the asynchronous read implementation. Writes to R0 are correctly blocked as observed by the unchanged 0x00000000 output on the R0 read.

## 5.4 Task 4 — Instruction Memory

The instruction memory testbench stepped the program counter through addresses 0 to 3 and then to the out-of-range value 0x00000020, verifying correct instruction fetch and boundary protection.

| PC Address | Instruction Fetched (hex) | Decoded Instruction | Pass? |
| --- | --- | --- | --- |
| 0x00000000 | 0x0D420000 | add $t0, $t1, $t2 | PASS |
| 0x00000001 | 0x17484000 | sub $t2, $t2, $t3 | PASS |
| 0x00000002 | 0x14160080 | and $t1, $t2, $t0 | PASS |
| 0x00000003 | 0x1EB51800 | or $t2, $t3, $t1 | PASS |
| 0x00000020 | 0x00000000 | NOP (out of range) | PASS |

The VCD shows instruction output changes at t=1 ns, 21 ns, 41 ns, and 61 ns as the PC increments. At t=81 ns when PC is set to 0x00000020, the instruction output correctly becomes 0x00000000, demonstrating the boundary protection logic.

## 5.5 Task 5 — Control Unit

The control unit testbench cycled through all eight opcodes, reading and asserting all ten control outputs for each. Selected assertions from the VCD waveform data are summarised below.

| Instruction | jump | branch | mem_read | mem_write | reg_write | alu_src | alu_op |
| --- | --- | --- | --- | --- | --- | --- | --- |
| add (R-type) | 0 | 0 | 0 | 0 | 1 | 0 | 10 |
| slti | 0 | 0 | 0 | 0 | 1 | 1 | 10 |
| j | 1 | 0 | 0 | 0 | 0 | 0 | 00 |
| jal | 1 | 0 | 0 | 0 | 1 | 0 | 00 |
| lw | 0 | 0 | 1 | 0 | 1 | 1 | 00 |
| sw | 0 | 0 | 0 | 1 | 0 | 1 | 00 |
| beq | 0 | 1 | 0 | 0 | 0 | 0 | 01 |
| addi | 0 | 0 | 0 | 0 | 1 | 1 | 11 |

All control signal assertions matched the expected values from the project specification table. Notably, the jal instruction correctly sets both jump=1 and reg_write=1, enabling the PC+4 return address to be written to R7. The beq instruction is the only one to assert branch=1, and sw is the only one to assert mem_write=1 while keeping reg_write=0, correctly preventing any register from being modified during a store operation.

6. Challenges and Solutions

## 6.1 Synchronous vs. Asynchronous Read in Register File

Initial design exploration considered making register reads synchronous. However, in a single-cycle processor the register file read must be available combinatorially within the same cycle so it can feed the ALU. The implementation was corrected to use asynchronous (combinational) read ports, resolving potential timing issues in the datapath.

## 6.2 32-bit Encoding with 3-bit Register Fields

The project specification references a 16-bit ISA for the theoretical description, while Task 4 requires 32-bit MIPS instruction encoding. The solution adopted 3-bit register address fields (sufficient for 8 registers) packed into the upper bits of a 32-bit word with zero-padding in the lower 14 bits. This allowed full 32-bit simulation while remaining consistent with the 8-register architectural constraint.

## 6.3 GHDL Elaboration and Library Scope

GHDL requires each entity to be elaborated individually before simulation. Early compilation attempts failed because testbench entities were compiled before their corresponding source entities. The solution was to enforce a strict compilation order in tasks.json: all source files first, then all testbenches, then elaboration and run steps per component.

## 6.4 R0 Write Protection

The MIPS convention that register 0 is hardwired to zero required explicit write-enable gating in the register file. The implementation gates the write with the condition reg_write_dest /= '000', preventing any write to R0 even when reg_write_en is asserted.

7. Conclusion and Future Work

This project successfully implemented and verified all five core components of a 32-bit single-cycle MIPS processor in VHDL. The Data Memory, ALU, Register File, Instruction Memory, and Control Unit were each designed with clean entity-architecture separation, thoroughly commented, and validated against all test cases specified in Tasks 1 through 5.

The simulation results confirm correct arithmetic (ADD, SUB, AND, OR, SLT), correct memory read/write with data retention, correct register file dual-port reads with R0 hardwiring, accurate instruction fetch with boundary protection, and correct control signal generation for all eight opcodes including jump-and-link.

Potential areas for future extension include:

- Integrating all five components into a complete single-cycle datapath with a program counter and PC adder.

- Adding pipeline registers to extend the design to a 5-stage MIPS pipeline.

- Extending the instruction set to support additional MIPS instructions such as sll, srl, lui, and nor.

- Targeting FPGA synthesis using Xilinx Vivado or Intel Quartus to validate timing on real hardware.

- Implementing a hazard detection unit and forwarding unit for the pipelined version.

8. References

[1] D. A. Patterson and J. L. Hennessy, Computer Organization and Design: The Hardware/Software Interface, 5th ed. Morgan Kaufmann, 2014.

[2] P. J. Ashenden, The Designer's Guide to VHDL, 3rd ed. Morgan Kaufmann, 2008.

[3] GHDL Open-Source VHDL Simulator, Version 4.0.0. Available: https://ghdl.github.io/ghdl/

[4] R. A. Sowah and R. Kuayi, CPEN 315 Final Project Brief, University of Ghana, Department of Computer Engineering, 2026.

[5] IEEE Standard VHDL Language Reference Manual, IEEE Std 1076-2008, IEEE, 2009.